CPCD40 Forklift Truck 

Features of the Diesel Forklift
1. With up to date appearance design, the CPCD40N series diesel forklift features streamline, dynamic and stylish appearance. 
2. Large sized covering part made by injection molding process makes the lift truck more elegant and user friendly. 
3. Long wheelbase design ensures higher longitudinal stability and more than 100kg lifting weight higher than the original forklift truck at the same lifting height. Load on the real axle is also reduced, prolonging the service life of steering wheel. 
4. Reduction of front and rear overhang size ensures smaller turning radius and lower width of passage for right angle stacking, thus improving warehouse use ratio. 
5. The diesel powered forklift can be equipped with Japanese OKAMURA, NISSAN and Chinese HC transmissions that come with best in class structure and reliable performance. Among which, the OKAMURA transmission applies the state of the art floating structure, ensuring completely flexible connection between engine and forklift frame, thus minimizing vibration. 

Technical Parameter 

	General 
	1 
	Product name 
	- 
	Forklift truck 

	
	2 
	Basic model 
	- 
	CPCD40 

	
	3 
	Type 
	- 
	RW5 
	RW19 
	RW35 
	RG24 

	
	4 
	Power type 
	- 
	Diesel 

	
	5 
	Rated load 
	kg 
	4000 

	
	6 
	Load center 
	mm 
	500 

	Characteristics & Dimensions of Forklift Truck 
	7 
	Lifting height 
	mm 
	3000 

	
	8 
	Free lift height 
	mm 
	160 

	
	9 
	Fork size 
	L×W×T 
	mm 
	1070×122×50 

	
	10 
	Mast tilt angle 
	F/R 
	degree 
	6/12 

	
	11 
	Fork overhang (Wheel center to fork surface) 
	mm 
	550 

	
	12 
	Rear overhang 
	mm 
	520 

	
	13 
	Overall dimensions 
	Length (To surface of fork) 
	mm 
	3070 

	
	14 
	
	Overall width 
	mm 
	1395 

	
	15 
	
	Mast closed height 
	mm 
	2245 

	
	16 
	
	Mast extended height 
	mm 
	4170 

	
	17 
	
	Overhead guard height 
	mm 
	2250(2290*) 

	
	18 
	Turning radius (Outside) 
	mm 
	2690 

	
	19 
	Min. width of right angle stacking aisle 
(Including load length and clearance) 
	mm 
	3240 

	Performance of Forklift Truck 
	20 
	Speed 
	Travel speed (No load) 
	km/h 
	0-25 
	0-26 
	0-24 
	0-26 

	
	21 
	
	Lifting speed (Full load) 
	mm/sec 
	440 
	460 
	430 
	440 

	
	22 
	
	Lowering speed (Full load) 
	mm/sec 
	500 

	
	23 
	Max. drawbar pull (Full load) 
	N 
	38200 
	35900 
	35000 
	37200 

	
	24 
	Max. gradeability (Full load) 
	% 
	20 

	Chassis 
	25 
	Tyre 
	Front 
	mm 
	8.25-15-14PR 

	
	26 
	
	Rear 
	mm 
	7.00-12-12PR 

	
	27 
	Tread 
	Front 
	mm 
	1160 

	
	28 
	
	Rear 
	mm 
	1130 

	
	29 
	Wheelbase 
	mm 
	2000 

	
	30 
	Ground clearance 
	Lowest point 
	- 
	170 

	
	31 
	
	Wheelbase center 
	- 
	230 

	Weight of Forklift Truck 
	32 
	Dead weight 
	kg 
	6300 

	
	33 
	Weight distribution 
	Full load 
	F/R 
	kg 
	9000/1300 

	
	34 
	
	No load 
	F/R 
	kg 
	2800/3500 

	Power & transmission 
	35 
	Battery 
	Voltage/Capacity (20HR) 
	V/AH 
	24/70/20h 

	
	36 
	Engine 
	Model 
	- 
	B3.3T 
	TD42 
	1104 
	4108 

	
	37 
	
	Manufacturer 
	- 
	COMMINS 
	UD 
	PERKINS 
	LUOTUO 

	
	38 
	
	Rated output/rpm 
	kW 
	60/2200 
	59/2300 
	56/2200 
	57/2200 

	
	39 
	
	Rated torque/rpm 
	N.m 
	292/1600 
	254/1800 
	265/1700 
	285/1400 

	
	40 
	
	Quantity of cylinder 
	- 
	4 
	6 
	4 
	4 

	
	41 
	
	Bore×stroke 
	mm 
	95×115 
	96×96 
	105×127 
	108×125 

	
	42 
	
	Emission standard 
	- 
	Stage II/Tirer2 
	- 
	Stage III 
	- 

	
	43 
	
	Displacement 
	cc 
	3300 
	4169 
	4410 
	4580 

	
	44 
	
	Fuel tank capacity 
	L 
	80 
	80 
	80 
	80 

	
	45 
	Transmissions 
	Manufacturer 
	- 
	CHINA 

	
	46 
	
	Type 
	- 
	Power shift 

	
	47 
	
	Gear 
	Forward/reverse 
	- 
	2/1 

	Others 
	48 
	Operating pressure 
	For attachments 
	MPa 
	18 

	Note: *With suspension seat or cabin.
1. We can manufacture the forklift truck with customized lifting height. (2-stage mast: 2.5m, 2.7m, 3.0m, 3.3m, 3.6m, 4m, and 4.5m. 3-stage mast: 4.3m, 4.5m, 4.8m, 5m, 5.5m, and 6m.)
2. This diesel forklift truck can be produced with attachment.
3. Data in the table is for standard model only. Please contact us for more details related to the high safety fork truck.
4. All specifications for the material handling equipment are subject to change without prior notice. 


